Unit : I

A computer network is a set of computers sharing resources located on or provided by network nodes. Computers use common communication protocols over digital interconnections to communicate with each other. These interconnections are made up of telecommunication network technologies based on physically wired, optical, and wireless radio-frequency methods that may be arranged in a variety of network topologies.

The nodes of a computer network can include personal computers, servers, networking hardware, or other specialized or general-purpose hosts. They are identified by network addresses and may have hostnames. Hostnames serve as memorable labels for the nodes and are rarely changed after initial assignment. Network addresses serve for locating and identifying the nodes by communication protocols such as the Internet Protocol.

Computer networks may be classified by many criteria, including the transmission medium used to carry signals, bandwidth, communications protocols to organize network traffic, the network size, the topology, traffic control mechanisms, and organizational intent.[citation needed]

Computer networks support many applications and services, such as access to the World Wide Web, digital video and audio, shared use of application and storage servers, printers and fax machines, and use of email and instant messaging applications.

Internet overview The Internet is a giant network of networks. • A network may include PCs, and other devices like servers or printers. • A network is connected through a communication channel.

• Early research was performed by the US Department of Defense in 1962. This research group established ARPAnet (Advanced Research Project Agency) in order to connect the US Defense Department network. What did the Internet come from? • Original aim was to create a network that would allow users of a research computer at one university to be able to ‘talk to’ research computers at other universities.

• A side benefit of ARPAnet’s design was that, because messages could be routed or rerouted in more than one direction, the network could continue to function even if parts of it were destroyed in the event of a military attack or other disaster.

• The users of the Internet took a direction of their own. History of the Internet

• The first long distance communication took place in 1965 between a computer in MIT and California

• In 1969, four computers clients were connected together via ARPAnet. How old is the Internet ?

• Leonard Kleinrock is accredited with the idea of packet switching, which describes how data can be sent across a network. The Ethernet was developed by Xerox during this period. This was inspired by Robert Metcalfe’s PhD on ‘packet networks’.

• An Ethernet is a protocol for describing how computers can be connected in a LAN (Local Area network).

• Through the use of Ethernet and ARPAnet the US were able to develop a working network.

• In the late 1970s and early 1980s other networks were developed, e.g. CSNET, USNET and BITNET. In 1973 Vint Cerf and Bob Kahn created the TCP/IP communication protocols.

• TCP/IP: Transfer Control Protocol/Internet Protocol is a set of rules that describe how computers can communicate over a network.

• To send information over the Internet, a computer packs data into Internet Protocol (IP) packets and labels them with the correct address. They are then sent across a packet switched interconnected network

Classification of Computer Networks

1.Based on Transmission Mode Transmission mode defines the direction of signal flow between two linked devices.

There are three types of transmission modes.  Simplex In simplex mode, the communication is unidirectional. Among the stations only one can transmit and the other can only receive

.  Half-Duplex In half-Duplex mode, the communication is bidirectional. In this both station can sent and receive but not at the same time.

Full-Duplex In Full-Duplex mode, both stations can transmit and receive simultaneously.

2. Based on Time in Transmission Type

• Synchronous Transmission In synchronous Transmission both the sender and the receiver use the same time cycle forthe transmission. We send bits one after another without start/stop bits or gaps. It is the responsibility of the receiver to group the bits. Bit stream is delivered with a fixed delay and given error rate. Each bit reaches the destination with the same time delay after leaving the source.

• Asynchronous Transmission In Asynchronous Transmission we send one start bit at the beginning and one stop bit at the end of each byte. There may be a gap between each byte. Bit stream is divided into packets. Packets are received with varying delays, so packets can arrive out of order. Some packets are not received correctly.

3. Based on Authentication

• Peer to Peer Connection In peer-to-peer networks, there are no dedicated servers. All the computers are equal and, therefore, are termed as peers. Normally, each computer functions as both a client and a server. No one can control the other computers.

• Server Based Connection Most networks have a dedicated server. A dedicated server is a computer on a network which functions as a server, and cannot be used as a client or a workstation. A dedicated server is optimized to service requests from network clients. A server can control the clients for its services.

4. Based on Geographical location

• Local Area Networks (LAN) LAN is a small high speed network. In LAN few numbers of systems are interconnected with networking device to create network. As the distance increases between the nodes or system it speed decreases. So it is limed to few meters only. Networks which cover close geographical area. LAN used to link the devices in a single office, building or campus. It provides high speeds over short distance. Systems are connecting directly to Network. The LAN is owned by private people.

• Wide Area Network (WAN) WAN is collection of network (or LAN). This network speed is less than the LAN network speed.WAN network connect systems indirectly. WAN spread over the world may be spread over more than one city country or continent. Systems in this network are connected indirectly. Generally WAN network are slower speed than LAN’s. The WAN network are owned or operated by network providers. If it is owned by a single owner then it is called Enterprise network. Often these types have combination of more than one topology.

• MAN (Metropolitan Area Network) Metropolitan area network is an extension of local area network to spread over the city. Itmay be a single network or a network in which more than one local area network canshare their resources.

5. Based on Reliability Reliability is maintained by authentication.

• Connection-oriented This type of communication establishes a session connection before data can be sent. This method is often called a "reliable" network service. It can guarantee that data will arrive in the same order.

• Connection less This type of communication does not require a session connection between sender and receiver for data transfer. The sender simply starts sending packets to the destination. A connectionless network provides minimal services.

Topology

Topology refers to physical layout including computers, cables, and other resources; it determines how components communicate with each other. Today’s network designs are based on three topologies:

• Bus consists of series of computers connected along a single cable segment • Star connects computers via central connection point or hub

• Ring connects computers to form a loop All computers, regardless of topology, communicate by addressing data to one or more computers and transmitting it across cable as electronic signals. Data is broken into packets and sent as electronic signals that travel on the cable. Only the computer to which the data is addressed accepts it. Protocol Protocols mean set of rules. It is a formal description of message formats and the rules two or more machines has follow to exchange messages. The key elements of a protocol are syntax, semantics and timing.

• Syntax Syntax refers to the structure or format of the data, meaning the order in which they arepresented.

• Semantics Semantics refers to the meaning of each section of bits.

• Timing Timing refers to when data should be sent and how fast it can be sent. Internetworking Technologies Internetworking Technologies tell how the Internet accommodating multiple underlying hardware technologies and how they are interconnected and formed the network, and set of communication standard which the network used to inter-operate. The lowercase internet means multiple networks connected together, using a commonprotocol suite. The uppercase Internet refers to the collection of hosts around the world that can communicate with each other using TCP/IP. While the Internet is an internet, the reverse is not true

Network Infrastructure or Transmission Infrastructure: Network infrastructure is divided into two parts. 1. Access Networks An access network is the part of a telecommunications network which connects end system to the first router or subscribers to their immediate service provider as shown in figure

- It is different from core network which connects all the routers to each other and ISP(Internet service provider). An access network may be a so-called local area network within a company or university, a dial telephone line with a modem, or a high-speed cable-based or phone-based access network. Figure 1 Network Infrastructure Access networks can be loosely divided into three categories:

- • Residential access networks, connecting a home end system into the network.

- • Institutional access networks, connecting an end system in a business or educational institution into the network.

- • Mobile access networks, connecting a mobile end system into the network Core Networks: Core network connects all the routers to each other and ISP (Internet service provider). It is a main back bone for internet. Core network uses circuit switching and packet switching for data transmission. ISPs:(Internet Service Provider) In internet bottom-to-top the hierarchy consists of end systems (PCs, workstations, etc.)connected to local Internet Service Providers (ISPs). The local ISPs are in turn connected to regional ISPs, which are in turn connected to national and international ISPs. The national and international ISPs are connected together at the highest tier in the hierarchy. Let's begin at the top of the hierarchy and work our way down. Residing at the very top of the hierarchy are the national ISPs, which are called National Backbone Provider (NBPs).

- The NBPs form independent backbone networks that span North America (and typically abroad as well). Just as there are multiple long-distance telephone companies in the USA, there are multiple NBPs that compete with each other for traffic and customers. The existing NBPs include internetMCI, SprintLink, PSINet, UUNet Technologies, and AGIS. The NBPs typically have high-bandwidth transmission links, with bandwidths ranging from

- 1.5 Mbps to 622 Mbps and higher. Each NBP also has numerous hubs which interconnect its links and at which regional ISPs can tap into the NBP. The NBPs themselves must be interconnected to each other. To see this, suppose one regional ISP, say MidWestnet, is connected to the MCI NBP and another regional ISP, say EastCoastnet, is connected to Sprint's NBP. How can traffic be sent from MidWestnet to EastCoastnet? The solution is to introduce switching centers, called Network Access Points (NAPs), which interconnect the NBPs, thereby allowing each regional ISP to pass traffic to any other regional ISP. To keep us all confused, some of the NAPs are not referred to as NAPs but instead as MAEs (Metropolitan Area Exchanges). Component of Internet: A network (or internet) is formed using

Hardware (or network device) and network software or Application and protocols.

Hardware or Network device:

- Hub: • It is uses to connect systems or nodes or networks. • It has direct connection to a node (point to point connection).

• It suffers from high collision of data, results to data loss.

• A hub takes data from input port and retransmits the input data on output port

2. Repeater:

• A repeater is a device which regenerates or amplifies the data or signal so that it can be travel to the other segment of cable.

• It is use to connect two networks that uses same technology and protocol

. • It does not filter or translate any data. • Work in physical layer. 3. Bridge: • It is used to connect two networks.

• It divides the collision domain based on number of ports or interface present in a bridge.

• It uses the packet switches that forward and filter the frames using LAN destination address.

• Bridge examines the destination address of frame and forwards it to the interface or port which leads to the destination.

• It uses the routing table for routing frame from one node to other using MAC address.

• It works in Data Link Layer.

4. Switch :

• It is similar to bridge. It has more number of interfaces as compared to bridge.

• It allows direct communication between the nodes.

• It works in Data Link Layer.

• It uses MAC address for data transmission and communication.

5. Router:

• It is used to connect different types of network (types- architecture/ Protocol).

• It work similar to bridge but it uses IP address for routing data.

• Router can't be used for connecting Systems.

• It works in Network Layer. 6. Gateways: Gateways make communication possible between systems that use different communication protocols, data formatting structures, languages and architectures. Gateways repackage data going from one system to another. Gateways are usually dedicated servers on a network and are task-specific. System, Software and Protocols: Basically two types of system are used in Internet

• Client system: User which access data from internet.

• Server System: Host data for users using HTML files. Software or Applications and protocols:

• Chat- IRC (Internet Relay Chat) is used for live discussions on the Internet.

• Ecommerce - Taking orders for products and services on the Internet.

• E-mail - Exchanging electronic letters, messages, and small files.

• FTP - File Transfer Protocol is the most common method of transferring files between computers via the Internet. • Hosting - Making information available to others on the Internet. • Search Engines - These tools are really a part of the World Wide Web and are often used when looking for information because the Web has grown so large and is without any inherent organizational structure. • Telnet - Creation of a dumb terminal session to a host computer in order to run software applications on the host system.

• World Wide Web - This is largest, fastest growing, part of the Internet, the part for which Internet browsers like Netscape’s Navigator and Microsoft’s Explorer were designed. Business is the leading factor fueling the rapid growth of the Web making information, advertising, and product ordering readily available to everyone with Web access.

• TCP/IP • Browser WAN Protocols

• Frame Relay Frame relay is used to connect large number of sites in the network because it is relatively inexpensive to do so. The service provider gives you a frame relay circuit and is charged for the amount of data and the bandwidth you use as oppose to T1 circuit that charges with a flat monthly rate whether you use partial bandwidth or the full bandwidth regardless. Frame relay is a high performance WAN protocol that operates at the Data Link layer and the Physical layer of the OSI model.

• Integrated Services Digital Network (ISDN) Integrated Services Digital Network (ISDN) is designed to run over existing telephone networks. It can deliver end to end digital service carrying voice and data. ISDN operates at OSI model, physical layer, data link layer and network layer. It can carry multimedia and graphics with all other voice, data services. ISDN supports all upper layer protocols and you can choose PPP, HDLC or LAPD as your encapsulation protocol. It has two offerings, Primary rate which is 23B+D channels. 23, 64 kbps and one 64kbps mainly used for signaling. The other is the Basic Rate which has 2B+D channels two 64kbps and one 16kbps. At data link layer ISDN supports two protocols; LAPB and LAPD. LAPB is used to mainly transfer data from upper layers and has three types of frames. I-Frames carry upper layer information and carries out sequencing, flow control, error detection and recovery. S- Frames carry control information for the I-frame. LAPD provides an additional multiplexing function to the upper layers enabling number of network entities to operate over a single physical access. Each individual link procedure acts independently of others. The multiplex procedure combines and distributes the data link channels according to the address information of the frame. Each link is associated with a specific Service Access Point (SAP), which is identified in the part of the address field.

• High Level Data Link Control (HDLC) High Level Data Link Control (HDLC) is a bit oriented data link layer frame protocol that has many versions similar to LAP, LAPB, and LAPD. CISCO routers default encapsulation is HDLC, but it is proprietary to CISCO. OSI model OSI (Open System Interconnection), developed by the International Organizationfor Standardization (ISO), was the solution designed to promote interoperabilitybetween vendors. It defines architecture for communications that support distributed processing. The OSI model describes the functions that allow systemsto communicate successfully over a network. Using what is called a layeredapproach, communications functions are broken down into seven distinct layers. Figure 2 Interaction between layers in OSI model. The seven layers, beginning with the bottom layer of the OSI model, are shown in figure 2.Routers are used as intermediate node to create a link between A and B end system. OSI model layers are dependent on each other. Each layer serves the upper layer and also depends upon the services from the lower layer. OSI model also provide the layer abstraction. Layers are dependent on each other for services but in terms of protocol they are independent. In each layer information is added into original data as header but in data link layer trailer is added into the data as shown in figure OSI Model Layer Layer

1: Physical Layer

▪ It defines the transmission of data across the communications medium and translation of binary data into signals.

▪ Mode of transmission over the link i.e Simplex or Half Duplex or Full Duplex ▪ It defines the transmission rate of bits per second. Figure 3 Exchange of data using OSI model Layer

2: Data Link Layer

▪ It divides the data into number of frames.

▪ It uses the MAC address for sending frames from one node to other.

▪ It provides flow control, error control and access control. Layer

3: Network Layer ▪ It divides data into number of packets. ▪ It uses IP address for routing packets to their destination. ▪ It provides end to end connection. Layer

4: Transport Layer ▪ It divides message into segments and also reassemble the segments to create original message.

▪ It can be either connection-oriented or connectionless.

▪ It uses service-point address or port address for process to process communication.

▪ Flow control and error control also provided by transport layer. Layer

5: Session Layer ▪ Session Layer establishes, maintains and synchronizes the interaction among communicatingsystems. Layer

6: Presentation Layer ▪ It is concerned with the syntax and semantics of the informationexchangedbetweentwo systems.

▪ It translates information from text/numeric into bit stream.

▪ It also encrypts the information for security purpose and compress the information to reduce the number of bits in the information. Layer

7: Application Layer ▪ It provides the interface to the end user and supports for services such as Email, file transfer and distributed information service. OSI Model and Protocol stack Layer Protocol Application HTTP, FTP, SMTP,TELNET Presentation JPG, GIF, MPEG, Session TCP 3-way Handshaking Transport TCP, UDP Network IP, IPX Data Link Ethernet, Token Ring, HDLC Physical X.21, RS-232, DS, DS3 TCP/IP model

• TCP/IP protocol suite was developed before the OSI model.

• TCP/IP is a set of protocols developed to allow cooperating computers to share resources across a network.

• In 1969 the Defense Advanced research projects Agency (DARPA) funded a research and development project to create an experimental packet switching network. This network is called ARPANET.

• In 1975 the ARPANET was converted from an experimental network to an operational network, and the responsibility for administering the network was given to the Defense Communication Agency .

Unit : II

The Physical Layer and the Data Link Layer are the first two, foundational layers of the seven-layer OSI model. Think of them as the groundwork for all network communication.

### Physical Layer (Layer 1)

This is the hardware layer. It's all about the physical connection and the transmission of raw, unstructured data – just a stream of bits (0s and 1s).

Think of it like this: Imagine laying down the actual cable (copper, fiber optic) or setting up a wireless connection (radio waves). You also need to decide on the type of electrical signals or light pulses you'll use to represent those 0s and 1s.

Key functions include:

- Physical Connection: Defining the physical interface, like the types of cables, connectors, and voltage levels.

- Data Transmission: Actually sending and receiving those raw bits over the physical medium.

- Signaling: Converting the digital data (bits) into signals suitable for the transmission medium (e.g., electrical signals, light pulses, radio waves).

- Bit Synchronization: Ensuring that the sender and receiver agree on the timing of each bit.

- Data Rate: Specifying how fast data is transmitted (bits per second).

- Topology: Defining the physical arrangement of devices (e.g., bus, star, ring, mesh).

- Transmission Mode: Determining the direction of data flow (simplex, half-duplex, full-duplex).

In a nutshell, the Physical Layer is concerned with the 'how' of physically moving data.

### Data Link Layer (Layer 2)

Building on the physical connection, the Data Link Layer is responsible for reliable transfer of data frames between two directly connected nodes. It takes the raw bitstream from the Physical Layer and structures it into meaningful units called frames.

Think of it like this: Once you have the physical connection, you need rules for how to package the data, address the devices on the local network, and make sure the data arrives correctly between those directly connected devices.

Key functions include:

- Framing: Taking the stream of bits from the Physical Layer and grouping them into frames, adding headers and trailers for control information.

- Physical Addressing (MAC Addressing): Adding the physical addresses (MAC addresses) of the sender and receiver to the frame. This is for local network communication.

- Error Control: Detecting and possibly correcting errors that occurred during transmission in the Physical Layer. Techniques like checksums and cyclic redundancy checks (CRC) are used.

- Flow Control: Managing the rate of data transmission between two nodes to prevent a fast sender from overwhelming a slow receiver.

- Media Access Control (MAC): In shared media networks (like some older forms of Ethernet or Wi-Fi), this sublayer determines how devices take turns to access the physical medium to avoid collisions.

- Logical Link Control (LLC): This sublayer provides an interface to the Network Layer (Layer 3) and handles tasks like flow and error control. (Note: In modern Ethernet networks, the LLC sublayer isn't always explicitly used for all protocols.)

Unit : III

The Network Layer and the Transport Layer are the next two crucial layers in the OSI model, building upon the foundation laid by the Physical and Data Link Layers. They handle the more complex aspects of moving data across networks.

### Network Layer (Layer 3)

This layer is all about routing and logical addressing. It's responsible for getting data packets from a source host to a destination host, even if they are on different networks.

Think of it like this: Imagine sending a letter across the country. The Network Layer is like the postal service. It figures out the best route, puts the correct address on the envelope (logical addressing), and makes sure your letter gets to the right city and state.

Key functions include:

- Logical Addressing (IP Addressing): Assigning unique logical addresses (IP addresses) to devices on the network. This allows for identification and routing across different networks.

- Routing: Determining the best path for data packets to travel from the source to the destination network. Routers operate at this layer.

- Packet Forwarding: Moving packets from an input interface to an output interface on a router based on the routing decisions.

- Fragmentation and Reassembly: If a packet is too large for a particular network's maximum transmission unit (MTU), the Network Layer can break it into smaller fragments at the source and reassemble them at the destination.

- Internetworking: Enabling communication between different types of networks.

In a nutshell, the Network Layer is concerned with the 'where' of data delivery – getting it to the right network.

### Transport Layer (Layer 4)

The Transport Layer focuses on end-to-end communication between applications running on different hosts. It provides services to the upper layers, ensuring reliable and efficient data transfer.

Think of it like this: Once your letter arrives in the correct city, the Transport Layer is like the specific instructions on the envelope (like "Attention: Mailroom") that ensure it gets to the right department or person within that building (the correct application).

Key functions include:

- Segmentation and Reassembly: Breaking down the data received from the upper layers into smaller units called segments (for TCP) or datagrams (for UDP) for transmission, and reassembling them at the destination.

- Service Point Addressing (Port Numbers): Using port numbers to identify specific applications or processes running on the hosts. This ensures that the data is delivered to the correct application.

- Connection Control:

  - Connection-oriented: Establishing a virtual connection between the source and destination before data transfer (like TCP). This provides reliability.

  - Connectionless: Sending data without establishing a connection beforehand (like UDP). This is faster but less reliable.

- Flow Control: Managing the rate of data transmission between the sender and receiver to prevent a fast sender from overwhelming a slow receiver.

- Error Control: Detecting and possibly retransmitting lost or corrupted segments to ensure reliable data delivery (primarily in TCP).

- Multiplexing and Demultiplexing: Allowing multiple applications on a host to share the same network connection (multiplexing at the sender) and directing incoming data to the correct application (demultiplexing at the receiver)

Unit : IV

The Application Layer is the topmost, seventh layer of the OSI reference model. It's the layer that directly interacts with end-user applications. Think of it as the window through which your software applications access the network to send and receive data.

Key functions and characteristics:

- Provides Network Services to Applications: This layer doesn't contain the user applications themselves, but it provides the protocols and services that these applications use to communicate over the network. For example, your web browser (an application) uses HTTP (an Application Layer protocol) to fetch web pages.

- User Interface to the Network: It's the layer closest to the end-user, providing a way for applications to utilize network resources.

- Application-Specific Protocols: A wide range of protocols operate at this layer, each designed for a specific purpose. These protocols define the rules and data formats that applications use to communicate.

- Focus on What the User Sees: Unlike the lower layers that deal with the technicalities of data transmission, the Application Layer is concerned with how the user interacts with the network through their software.

Think of it like this: When you use an application that needs network access (like sending an email or browsing a website), that application interacts with the Application Layer protocols to get the job done. The Application Layer then hands the data down to the lower layers for the actual transmission.

Common Application Layer Protocols:

You're likely familiar with many of these:

- HTTP (Hypertext Transfer Protocol): Used for web browsing.

- HTTPS (HTTP Secure): A secure version of HTTP.

- FTP (File Transfer Protocol): Used for transferring files.

- SMTP (Simple Mail Transfer Protocol): Used for sending emails.

- POP3 (Post Office Protocol version 3) & IMAP (Internet Message Access Protocol): Used for retrieving emails.

- DNS (Domain Name System): Translates domain names into IP addresses.

- DHCP (Dynamic Host Configuration Protocol): Assigns IP addresses 1 to devices on a network.

- SSH (Secure Shell): Provides secure remote access to computers.

- Telnet: Provides unsecure remote access to computers.

- SNMP (Simple Network Management Protocol): Used for managing network devices.